ATLAS AIRCRAFT TRUST   |   AIRCRAFT LEASING

DELIVERY CONDITION REPORT

Aircraft Delivery — N56789 / MSN 45912

Reference: DCR-N56789-2024-09-01

| Aircraft Type | MSN | Registration | Engines | Year |
| --- | --- | --- | --- | --- |
| Boeing 737 MAX 8 | 45912 | N56789 | CFM LEAP-1B27 | 2022 |

| Role | Party | Jurisdiction |
| --- | --- | --- |
| Lessor | Atlas Aircraft Trust | Ireland |
| Lessee | Nordic Skies AS | USA |
| Delivery Location | Dallas/Fort Worth Int'l Airport (DFW) | Texas, USA |

SIGNED ORIGINAL

Dated: 2024-09-01

This document is strictly confidential and is intended solely for the named parties. Any reproduction, disclosure, or distribution without the prior written consent of Atlas Aircraft Trust is strictly prohibited.

## 1. DELIVERY SUMMARY

| Parameter | Detail |
| --- | --- |
| Aircraft Type | Boeing 737 MAX 8 |
| MSN | 45912 |
| Registration | N56789 |
| Lessor | Atlas Aircraft Trust |
| Lessee | Nordic Skies AS |
| Delivery Date | 2024-09-01 |
| Delivery Location | Dallas/Fort Worth Intl Airport (DFW), Texas, USA |
| Inspector – Lessor | Thomas Kavanaugh (AME Lic. 123456), TK Technical Services |
| Inspector – Lessee | Robert A. Fitch (IA Cert. 654321), Director of Technical |
| Total FH at Delivery (TTSN) | 22,841 FH |
| Total FC at Delivery (TCSN) | 16,203 FC |
| FH since last C-Check | 4,102 FH |
| Date of last C-Check | 2022-03-14 |

## 2. ATA CHAPTER INSPECTION RESULTS

Inspection findings by ATA chapter. Items with STATUS = 'Discrepancy' are detailed in Section 9.

| ATA Chapter | Status | Inspector Notes |
| --- | --- | --- |
| ATA 05 – Time Limits/Maintenance Checks | Satisfactory | Per Mx Programme |
| ATA 06 – Dimensions and Areas | Satisfactory | Confirmed per ASM |
| ATA 07 – Lifting and Shoring | Satisfactory | All certified |
| ATA 10 – Parking and Mooring | Satisfactory | All tied down |
| ATA 11 – Placards and Markings | Satisfactory | All legible, compliant |
| ATA 12 – Servicing | Satisfactory | All fluids serviced |
| ATA 21 – Air Conditioning | Satisfactory | Functional test OK |
| ATA 22 – Auto Flight | Satisfactory | All AFCS tests pass |
| ATA 23 – Communications | Satisfactory | Slight wear in headliner lining |
| ATA 24 – Electrical Power | Satisfactory | All ELEC buses tested |
| ATA 25 – Equipment and Furnishings | Satisfactory | Seat covers minor scuff |
| ATA 26 – Fire Protection | Satisfactory | All detectors tested |
| ATA 27 – Flight Controls | Satisfactory | All surfaces checked |
| ATA 28 – Fuel | Satisfactory | Sealants checked |
| ATA 29 – Hydraulic Power | Satisfactory | Sys pressure normal |
| ATA 30 – Ice and Rain Protection | Satisfactory | Boots/weep holes OK |
| ATA 31 – Instruments | Satisfactory | EFIS/EICAS checked |
| ATA 32 – Landing Gear | Satisfactory | Tyre wear: 60% remain |
| ATA 33 – Lights | Satisfactory | Lighting functional |
| ATA 34 – Navigation | Satisfactory | Nav DB current |
| ATA 35 – Oxygen | Satisfactory | Qty checked |
| ATA 36 – Pneumatic | Satisfactory | All valves OK |
| ATA 38 – Water and Waste | Satisfactory | No leaks, potable water OK |
| ATA 45 – Central Maintenance System | Satisfactory | CMS no active faults |
| ATA 46 – Information Systems | Satisfactory | IFE functional minor screen fault |
| ATA 49 – APU | Satisfactory | APU start-stop, load test |
| ATA 51 – Structures | Satisfactory | No disbonds observed |
| ATA 52 – Doors | Satisfactory | All latches tested |
| ATA 53 – Fuselage | Satisfactory | No visible cracks |
| ATA 54 – Nacelles / Pylons | Satisfactory | Cowl hinges OK |
| ATA 55 – Stabilizers | Satisfactory | Elevator trim checked |
| ATA 56 – Windows | Satisfactory | All panels intact |
| ATA 57 – Wings | Satisfactory | No disbonds |
| ATA 71 – Power Plant | Satisfactory | ENG 1 start OK |
| ATA 72 – Engine | Satisfactory | Blade FOD check negative |
| ATA 73 – Engine Fuel and Control | Satisfactory | Fuel scheduling normal |
| ATA 74 – Ignition | Satisfactory | Exciter test pass |
| ATA 75 – Air | Satisfactory | Bleed valve tested |
| ATA 76 – Engine Controls | Discrepancy | Thrust lever friction HIGH — see Discrepancy DCR-001 |
| ATA 80 – Starting | Satisfactory | Start cycle test pass |

## 3. ENGINE STATUS — ENGINE NO. 1 (ESN ESN-100201)

| Parameter | Value |
| --- | --- |
| Engine Type | CFM LEAP-1B27 |
| ESN | ESN-100201 |
| Position | No. 1 (Left / Wing Pylon 1) |
| TTSN | 18,203 FH |
| TCSN | 14,822 FC |
| TSLSV (FH) | 4,102 FH / 2,918 FC |
| Last SV Date | 2022-03-18 |
| Last SV Type | Performance Restoration (PR) — CFM Approved |
| Last SV Facility | Delta TechOps, Atlanta GA — FAA OM1R074L / EASA 145.0049 |
| EGT Margin at Delivery | 38°C (minimum 30°C) — PASS |
| Oil Consumption | 0.18 qt/hr (limit 0.50 qt/hr) — PASS |
| Vibration N1/N2 | N1: 0.4 IPS / N2: 0.3 IPS — PASS |

### 3.1 Engine No. 1 — LLP Status

| Module | P/N | Description | Life | TSN | TSO | Remaining | % Left | Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fan | 1794M90P01 | Fan Disk | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| Fan | 1794M91P01 | Fan Blade (set of 24) | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| Fan | 1794M92P01 | Fan Case | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPC (Booster) | 1795M10P01 | Booster Disk Stage 1 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| LPC (Booster) | 1795M11P01 | Booster Disk Stage 2 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| LPC (Booster) | 1795M12P01 | Booster Disk Stage 3 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| LPC (Booster) | 1795M13P01 | Booster Spool | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| HPC | 1796M01P01 | HPC Disk Stage 3 | 15000 | 14822 | 14822 | 178 | 1.2% | ⚠ WARNING |
| HPC | 1796M02P01 | HPC Disk Stage 4 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| HPC | 1796M03P01 | HPC Disk Stage 5 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| HPC | 1796M04P01 | HPC Disk Stage 6 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| HPC | 1796M05P01 | HPC Disk Stage 7 | 25000 | 14822 | 14822 | 10178 | 40.7% | OK |
| HPC | 1796M06P01 | HPC Disk Stage 8 | 25000 | 14822 | 14822 | 10178 | 40.7% | OK |
| HPC | 1796M07P01 | HPC Disk Stage 9 | 25000 | 14822 | 14822 | 10178 | 40.7% | OK |
| HPC | 1796M08P01 | HPC Spool (3-4) | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| HPC | 1796M09P01 | HPC Spool (5-9) | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| HPT | 1797M01P01 | HPT Disk Stage 1 | 15000 | 14822 | 14822 | 178 | 1.2% | ⚠ WARNING |
| HPT | 1797M02P01 | HPT Disk Stage 2 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| HPT | 1797M03P01 | HPT Stage 1 Blade (set) | 10000 | 14822 | 2500 | 7500 | 75.0% | OK |
| HPT | 1797M04P01 | HPT Stage 2 Blade (set) | 10000 | 14822 | 2500 | 7500 | 75.0% | OK |
| LPT | 1798M01P01 | LPT Disk Stage 4 | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPT | 1798M02P01 | LPT Disk Stage 5 | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPT | 1798M03P01 | LPT Disk Stage 6 | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPT | 1798M04P01 | LPT Disk Stage 7 | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPT | 1798M05P01 | LPT Shaft | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |

## 4. ENGINE STATUS — ENGINE NO. 2 (ESN ESN-100202)

| Parameter | Value |
| --- | --- |
| ESN | ESN-100202 |
| Position | No. 2 (Right / Wing Pylon 2) |
| TTSN | 17,984 FH |
| TCSN | 14,618 FC |
| TSLSV | 3,841 FH / 2,714 FC |
| Last SV Date | 2022-05-04 |
| Last SV Facility | HAECO Xiamen — FAA MR7R078M / EASA 145.4170 |
| EGT Margin | 42°C — PASS |
| Oil Consumption | 0.21 qt/hr — PASS |
| Vibration | N1: 0.5 IPS / N2: 0.4 IPS — PASS |

| Module | P/N | Description | Life | TSN | TSO | Remaining | % Left | Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fan | 1794M90P01 | Fan Disk | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| Fan | 1794M91P01 | Fan Blade (set of 24) | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| Fan | 1794M92P01 | Fan Case | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPC (Booster) | 1795M10P01 | Booster Disk Stage 1 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| LPC (Booster) | 1795M11P01 | Booster Disk Stage 2 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| LPC (Booster) | 1795M12P01 | Booster Disk Stage 3 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| LPC (Booster) | 1795M13P01 | Booster Spool | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| HPC | 1796M01P01 | HPC Disk Stage 3 | 15000 | 14618 | 14618 | 382 | 2.5% | ⚠ WARNING |
| HPC | 1796M02P01 | HPC Disk Stage 4 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| HPC | 1796M03P01 | HPC Disk Stage 5 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| HPC | 1796M04P01 | HPC Disk Stage 6 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| HPC | 1796M05P01 | HPC Disk Stage 7 | 25000 | 14618 | 14618 | 10382 | 41.5% | OK |
| HPC | 1796M06P01 | HPC Disk Stage 8 | 25000 | 14618 | 14618 | 10382 | 41.5% | OK |
| HPC | 1796M07P01 | HPC Disk Stage 9 | 25000 | 14618 | 14618 | 10382 | 41.5% | OK |
| HPC | 1796M08P01 | HPC Spool (3-4) | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| HPC | 1796M09P01 | HPC Spool (5-9) | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| HPT | 1797M01P01 | HPT Disk Stage 1 | 15000 | 14618 | 14618 | 382 | 2.5% | ⚠ WARNING |
| HPT | 1797M02P01 | HPT Disk Stage 2 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| HPT | 1797M03P01 | HPT Stage 1 Blade (set) | 10000 | 14618 | 14618 | -4618 | -46.2% | ⚠ WARNING |
| HPT | 1797M04P01 | HPT Stage 2 Blade (set) | 10000 | 14618 | 14618 | -4618 | -46.2% | ⚠ WARNING |
| LPT | 1798M01P01 | LPT Disk Stage 4 | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPT | 1798M02P01 | LPT Disk Stage 5 | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPT | 1798M03P01 | LPT Disk Stage 6 | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPT | 1798M04P01 | LPT Disk Stage 7 | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPT | 1798M05P01 | LPT Shaft | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |

## 5. APU STATUS

| Parameter | Value |
| --- | --- |
| APU Model | Honeywell 131-9(B) |
| APU S/N | APU-591201 |
| Total APU-H Since New | 12,441 APU-H |
| Hours Since Last HSI | 2,108 APU-H |
| Last HSI Date | 2022-09-15 |
| Last HSI Facility | Honeywell Repair Station, Phoenix AZ — FAA OW1R089K |
| Cold Start Test | 22 sec to idle (ambient 18°C) — PASS |
| Bleed Air Load Test | 36 lb/min at sea level — PASS |
| APU EGT at 100% load | 508°C (limit 685°C) — PASS |

## 6. LANDING GEAR STATUS

| Gear | Serial No. | Overhaul Date | Facility | FH Since OH | FC Since OH | Next OH Due |
| --- | --- | --- | --- | --- | --- | --- |
| NLG | NLG-5912 | 2019-08-22 | AGES, Mexico City | 12,204 FH | 9,114 FC | 2029-08-22 |
| LH MLG | LH-5912 | 2020-02-14 | Messier Services, London Gatwick | 10,803 FH | 8,021 FC | 2030-02-14 |
| RH MLG | RH-5912 | 2020-02-14 | Messier Services, London Gatwick | 10,803 FH | 8,021 FC | 2030-02-14 |

## 7. CABIN INTERIOR SURVEY

| Area | Condition | Defects Noted | Action Required |
| --- | --- | --- | --- |
| Flight Deck | Good | None | None |
| Forward Galley (G1) | Good | Slight wear on trolley guides | Monitor |
| Business Class (16 seats) | Good | Seat 2A: armrest rattle | Rectify before service |
| Economy Class (132 seats) | Fair | Seat 14F: IFE screen blank; Seat 22B: pocket torn | Rectify IFE; replace pocket |
| Aft Galley (G2) | Good | None | None |
| Forward Lavatory | Fair | Waste flap hinge stiff | Rectify |
| Mid / Aft Lavatories (L2–L5) | Good | None | None |
| Overhead Bins | Good | Minor scuffing on door edges | Acceptable |
| Carpet – Forward | Good | No staining | None |
| Carpet – Economy | Fair | Minor wear rows 15–18 | Acceptable per Return Conditions |
| Sidewall Panels | Good | 2 panels with hairline cracks | Monitor |
| Emergency Equipment | Serviceable | All counts correct; no expiry within 12 months | None |

## 8. EXTERIOR CONDITION

| Zone | Condition | Noted Defects |
| --- | --- | --- |
| Nose / Radome | Good | Paint chip 1.5 cm at lower radome — no structural damage |
| Forward Fuselage (Sec 41/43) | Good | None |
| Mid Fuselage (Sec 44/46) | Good | Door R2 outer seal slight extrusion — serviceable |
| Aft Fuselage (Sec 46/48) | Good | None |
| Tail Section / APU Bay | Good | None |
| Vertical Stabiliser | Good | None |
| Horizontal Stabilisers | Good | None |
| Left Wing | Good | 2 small paint scuffs (approx 3×5 cm total) |
| Right Wing | Good | None |
| Engine No. 1 Nacelle | Good | None |
| Engine No. 2 Nacelle | Good | None |
| Overall Paint | Good | ~5% minor wear; no primer visible; no missing >2 cm |

## 9. DISCREPANCY LOG

| Ref | ATA | Description | Responsibility | Resolution |
| --- | --- | --- | --- | --- |
| DCR-001 | ATA 76 | Engine No.1 thrust lever friction high (8 lbf vs 6 lbf nominal) | Lessor to rectify pre-delivery | Adjust cable tension per AMM 76-11-00 |
| DCR-002 | ATA 25 | Seat 2A armrest rattle — Business Class | Lessee within 30 days | Replace armrest fitting; MEL Cat C |
| DCR-003 | ATA 25 | Seat 14F IFE screen blank | Lessee within 30 days | Replace IFE LRU |
| DCR-004 | ATA 25 | Seat 22B seatback pocket torn | Lessee within 60 days | Replace pocket fabric |
| DCR-005 | ATA 38 | Forward lavatory waste flap hinge stiff | Lessee within 30 days | Lubricate/replace hinge per AMM |
| DCR-006 | ATA 33 | Row 21 reading light inoperable — open MEL Cat C | Lessee: open MEL acknowledged | Replace at next A-Check |

## 10. OPEN MEL ITEMS AT DELIVERY

| MEL Ref | ATA | Description | Cat | Interval Expires |
| --- | --- | --- | --- | --- |
| MEL-23-001 | ATA 23 | SELCAL code channel 4 inoperable | B | 10 days from delivery |
| MEL-25-007 | ATA 25 | Seat 14F IFE screen blank | C | 30 days from delivery |
| MEL-33-012 | ATA 33 | Row 21 reading light inoperable | C | 120 days from delivery |

## 11. TECHNICAL RECORDS COMPLETENESS

| Record | Present | Complete | Comments |
| --- | --- | --- | --- |
| Aircraft Logbook | Yes | Yes | All entries legible and continuous |
| Engine No. 1 Logbook | Yes | Yes | Shop visit records attached |
| Engine No. 2 Logbook | Yes | Yes | Shop visit records attached |
| APU Logbook | Yes | Yes | All entries complete |
| Component Status List | Yes | Yes | Excel format; as at delivery date |
| LLP Tracking Sheets (Eng 1 & 2) | Yes | Yes | Disc sheets attached |
| AD Compliance Record | Yes | Yes | All ADs listed; repetitive due dates confirmed |
| SB Status Record | Yes | Yes | Alert SBs highlighted |
| C-Check Work Package | Yes | Yes | 1,847 work cards archived |
| Engine SV Reports | Yes | Yes | Delta TechOps (Eng 1), HAECO Xiamen (Eng 2) |
| Weight & Balance Report | Yes | Yes | Current; dated within 12 months |
| STCs / FAA 337s | Yes | Yes | IFE STC SA11801SW, SATCOM STC SA10912SW |

IN WITNESS WHEREOF the parties have executed this document as of the date first written above.

| LESSOR REPRESENTATIVE | LESSEE REPRESENTATIVE |
| --- | --- |
| By: _________________________ | By: _________________________ |
| Name: Thomas Kavanaugh | Name: Robert A. Fitch |
| Title: Technical Inspector | Title: Director of Technical |
| Date: 2024-09-01 | Date: 2024-09-01 |