GLOBAL WING FINANCE BV   |   AIRCRAFT LEASING

DELIVERY CONDITION REPORT

Aircraft Delivery — N67890 / MSN 7203

Reference: DCR-N67890-2022-11-01

| Aircraft Type | MSN | Registration | Engines | Year |
| --- | --- | --- | --- | --- |
| Airbus A319-100 | 7203 | N67890 | CFM56-5B5/P | 2016 |

| Role | Party | Jurisdiction |
| --- | --- | --- |
| Lessor | Global Wing Finance BV | Ireland |
| Lessee | Coastal Charter Services Ltd. | USA |
| Delivery Location | Dallas/Fort Worth Int'l Airport (DFW) | Texas, USA |

SIGNED ORIGINAL

Dated: 2022-11-01

This document is strictly confidential and is intended solely for the named parties. Any reproduction, disclosure, or distribution without the prior written consent of Global Wing Finance BV is strictly prohibited.

## 1. DELIVERY SUMMARY

| Parameter | Detail |
| --- | --- |
| Aircraft Type | Airbus A319-100 |
| MSN | 7203 |
| Registration | N67890 |
| Lessor | Global Wing Finance BV |
| Lessee | Coastal Charter Services Ltd. |
| Delivery Date | 2022-11-01 |
| Delivery Location | Dallas/Fort Worth Intl Airport (DFW), Texas, USA |
| Inspector – Lessor | Thomas Kavanaugh (AME Lic. 123456), TK Technical Services |
| Inspector – Lessee | Robert A. Fitch (IA Cert. 654321), Director of Technical |
| Total FH at Delivery (TTSN) | 22,841 FH |
| Total FC at Delivery (TCSN) | 16,203 FC |
| FH since last C-Check | 4,102 FH |
| Date of last C-Check | 2022-03-14 |

## 2. ATA CHAPTER INSPECTION RESULTS

Inspection findings by ATA chapter. Items with STATUS = 'Discrepancy' are detailed in Section 9.

| ATA Chapter | Status | Inspector Notes |
| --- | --- | --- |
| ATA 05 – Time Limits/Maintenance Checks | Satisfactory | Per Mx Programme |
| ATA 06 – Dimensions and Areas | Satisfactory | Confirmed per ASM |
| ATA 07 – Lifting and Shoring | Satisfactory | All certified |
| ATA 10 – Parking and Mooring | Satisfactory | All tied down |
| ATA 11 – Placards and Markings | Satisfactory | All legible, compliant |
| ATA 12 – Servicing | Satisfactory | All fluids serviced |
| ATA 21 – Air Conditioning | Satisfactory | Functional test OK |
| ATA 22 – Auto Flight | Satisfactory | All AFCS tests pass |
| ATA 23 – Communications | Satisfactory | Slight wear in headliner lining |
| ATA 24 – Electrical Power | Satisfactory | All ELEC buses tested |
| ATA 25 – Equipment and Furnishings | Satisfactory | Seat covers minor scuff |
| ATA 26 – Fire Protection | Satisfactory | All detectors tested |
| ATA 27 – Flight Controls | Satisfactory | All surfaces checked |
| ATA 28 – Fuel | Satisfactory | Sealants checked |
| ATA 29 – Hydraulic Power | Satisfactory | Sys pressure normal |
| ATA 30 – Ice and Rain Protection | Satisfactory | Boots/weep holes OK |
| ATA 31 – Instruments | Satisfactory | EFIS/EICAS checked |
| ATA 32 – Landing Gear | Satisfactory | Tyre wear: 60% remain |
| ATA 33 – Lights | Satisfactory | Lighting functional |
| ATA 34 – Navigation | Satisfactory | Nav DB current |
| ATA 35 – Oxygen | Satisfactory | Qty checked |
| ATA 36 – Pneumatic | Satisfactory | All valves OK |
| ATA 38 – Water and Waste | Satisfactory | No leaks, potable water OK |
| ATA 45 – Central Maintenance System | Satisfactory | CMS no active faults |
| ATA 46 – Information Systems | Satisfactory | IFE functional minor screen fault |
| ATA 49 – APU | Satisfactory | APU start-stop, load test |
| ATA 51 – Structures | Satisfactory | No disbonds observed |
| ATA 52 – Doors | Satisfactory | All latches tested |
| ATA 53 – Fuselage | Satisfactory | No visible cracks |
| ATA 54 – Nacelles / Pylons | Satisfactory | Cowl hinges OK |
| ATA 55 – Stabilizers | Satisfactory | Elevator trim checked |
| ATA 56 – Windows | Satisfactory | All panels intact |
| ATA 57 – Wings | Satisfactory | No disbonds |
| ATA 71 – Power Plant | Satisfactory | ENG 1 start OK |
| ATA 72 – Engine | Satisfactory | Blade FOD check negative |
| ATA 73 – Engine Fuel and Control | Satisfactory | Fuel scheduling normal |
| ATA 74 – Ignition | Satisfactory | Exciter test pass |
| ATA 75 – Air | Satisfactory | Bleed valve tested |
| ATA 76 – Engine Controls | Discrepancy | Thrust lever friction HIGH — see Discrepancy DCR-001 |
| ATA 80 – Starting | Satisfactory | Start cycle test pass |

## 3. ENGINE STATUS — ENGINE NO. 1 (ESN ESN-334455)

| Parameter | Value |
| --- | --- |
| Engine Type | CFM56-5B5/P |
| ESN | ESN-334455 |
| Position | No. 1 (Left / Wing Pylon 1) |
| TTSN | 18,203 FH |
| TCSN | 14,822 FC |
| TSLSV (FH) | 4,102 FH / 2,918 FC |
| Last SV Date | 2022-03-18 |
| Last SV Type | Performance Restoration (PR) — CFM Approved |
| Last SV Facility | Delta TechOps, Atlanta GA — FAA OM1R074L / EASA 145.0049 |
| EGT Margin at Delivery | 38°C (minimum 30°C) — PASS |
| Oil Consumption | 0.18 qt/hr (limit 0.50 qt/hr) — PASS |
| Vibration N1/N2 | N1: 0.4 IPS / N2: 0.3 IPS — PASS |

### 3.1 Engine No. 1 — LLP Status

| Module | P/N | Description | Life | TSN | TSO | Remaining | % Left | Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fan | 1794M90P01 | Fan Disk | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| Fan | 1794M91P01 | Fan Blade (set of 24) | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| Fan | 1794M92P01 | Fan Case | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPC (Booster) | 1795M10P01 | Booster Disk Stage 1 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| LPC (Booster) | 1795M11P01 | Booster Disk Stage 2 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| LPC (Booster) | 1795M12P01 | Booster Disk Stage 3 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| LPC (Booster) | 1795M13P01 | Booster Spool | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| HPC | 1796M01P01 | HPC Disk Stage 3 | 15000 | 14822 | 14822 | 178 | 1.2% | ⚠ WARNING |
| HPC | 1796M02P01 | HPC Disk Stage 4 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| HPC | 1796M03P01 | HPC Disk Stage 5 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| HPC | 1796M04P01 | HPC Disk Stage 6 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| HPC | 1796M05P01 | HPC Disk Stage 7 | 25000 | 14822 | 14822 | 10178 | 40.7% | OK |
| HPC | 1796M06P01 | HPC Disk Stage 8 | 25000 | 14822 | 14822 | 10178 | 40.7% | OK |
| HPC | 1796M07P01 | HPC Disk Stage 9 | 25000 | 14822 | 14822 | 10178 | 40.7% | OK |
| HPC | 1796M08P01 | HPC Spool (3-4) | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| HPC | 1796M09P01 | HPC Spool (5-9) | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| HPT | 1797M01P01 | HPT Disk Stage 1 | 15000 | 14822 | 14822 | 178 | 1.2% | ⚠ WARNING |
| HPT | 1797M02P01 | HPT Disk Stage 2 | 20000 | 14822 | 14822 | 5178 | 25.9% | OK |
| HPT | 1797M03P01 | HPT Stage 1 Blade (set) | 10000 | 14822 | 2500 | 7500 | 75.0% | OK |
| HPT | 1797M04P01 | HPT Stage 2 Blade (set) | 10000 | 14822 | 2500 | 7500 | 75.0% | OK |
| LPT | 1798M01P01 | LPT Disk Stage 4 | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPT | 1798M02P01 | LPT Disk Stage 5 | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPT | 1798M03P01 | LPT Disk Stage 6 | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPT | 1798M04P01 | LPT Disk Stage 7 | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |
| LPT | 1798M05P01 | LPT Shaft | 30000 | 14822 | 14822 | 15178 | 50.6% | OK |

## 4. ENGINE STATUS — ENGINE NO. 2 (ESN ESN-334456)

| Parameter | Value |
| --- | --- |
| ESN | ESN-334456 |
| Position | No. 2 (Right / Wing Pylon 2) |
| TTSN | 17,984 FH |
| TCSN | 14,618 FC |
| TSLSV | 3,841 FH / 2,714 FC |
| Last SV Date | 2022-05-04 |
| Last SV Facility | HAECO Xiamen — FAA MR7R078M / EASA 145.4170 |
| EGT Margin | 42°C — PASS |
| Oil Consumption | 0.21 qt/hr — PASS |
| Vibration | N1: 0.5 IPS / N2: 0.4 IPS — PASS |

| Module | P/N | Description | Life | TSN | TSO | Remaining | % Left | Flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fan | 1794M90P01 | Fan Disk | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| Fan | 1794M91P01 | Fan Blade (set of 24) | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| Fan | 1794M92P01 | Fan Case | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPC (Booster) | 1795M10P01 | Booster Disk Stage 1 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| LPC (Booster) | 1795M11P01 | Booster Disk Stage 2 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| LPC (Booster) | 1795M12P01 | Booster Disk Stage 3 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| LPC (Booster) | 1795M13P01 | Booster Spool | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| HPC | 1796M01P01 | HPC Disk Stage 3 | 15000 | 14618 | 14618 | 382 | 2.5% | ⚠ WARNING |
| HPC | 1796M02P01 | HPC Disk Stage 4 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| HPC | 1796M03P01 | HPC Disk Stage 5 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| HPC | 1796M04P01 | HPC Disk Stage 6 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| HPC | 1796M05P01 | HPC Disk Stage 7 | 25000 | 14618 | 14618 | 10382 | 41.5% | OK |
| HPC | 1796M06P01 | HPC Disk Stage 8 | 25000 | 14618 | 14618 | 10382 | 41.5% | OK |
| HPC | 1796M07P01 | HPC Disk Stage 9 | 25000 | 14618 | 14618 | 10382 | 41.5% | OK |
| HPC | 1796M08P01 | HPC Spool (3-4) | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| HPC | 1796M09P01 | HPC Spool (5-9) | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| HPT | 1797M01P01 | HPT Disk Stage 1 | 15000 | 14618 | 14618 | 382 | 2.5% | ⚠ WARNING |
| HPT | 1797M02P01 | HPT Disk Stage 2 | 20000 | 14618 | 14618 | 5382 | 26.9% | OK |
| HPT | 1797M03P01 | HPT Stage 1 Blade (set) | 10000 | 14618 | 14618 | -4618 | -46.2% | ⚠ WARNING |
| HPT | 1797M04P01 | HPT Stage 2 Blade (set) | 10000 | 14618 | 14618 | -4618 | -46.2% | ⚠ WARNING |
| LPT | 1798M01P01 | LPT Disk Stage 4 | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPT | 1798M02P01 | LPT Disk Stage 5 | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPT | 1798M03P01 | LPT Disk Stage 6 | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPT | 1798M04P01 | LPT Disk Stage 7 | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |
| LPT | 1798M05P01 | LPT Shaft | 30000 | 14618 | 14618 | 15382 | 51.3% | OK |

## 5. APU STATUS

| Parameter | Value |
| --- | --- |
| APU Model | Honeywell 131-9(B) |
| APU S/N | APU-720301 |
| Total APU-H Since New | 12,441 APU-H |
| Hours Since Last HSI | 2,108 APU-H |
| Last HSI Date | 2022-09-15 |
| Last HSI Facility | Honeywell Repair Station, Phoenix AZ — FAA OW1R089K |
| Cold Start Test | 22 sec to idle (ambient 18°C) — PASS |
| Bleed Air Load Test | 36 lb/min at sea level — PASS |
| APU EGT at 100% load | 508°C (limit 685°C) — PASS |

## 6. LANDING GEAR STATUS

| Gear | Serial No. | Overhaul Date | Facility | FH Since OH | FC Since OH | Next OH Due |
| --- | --- | --- | --- | --- | --- | --- |
| NLG | NLG-7203 | 2019-08-22 | AGES, Mexico City | 12,204 FH | 9,114 FC | 2029-08-22 |
| LH MLG | LH-7203 | 2020-02-14 | Messier Services, London Gatwick | 10,803 FH | 8,021 FC | 2030-02-14 |
| RH MLG | RH-7203 | 2020-02-14 | Messier Services, London Gatwick | 10,803 FH | 8,021 FC | 2030-02-14 |

## 7. CABIN INTERIOR SURVEY

| Area | Condition | Defects Noted | Action Required |
| --- | --- | --- | --- |
| Flight Deck | Good | None | None |
| Forward Galley (G1) | Good | Slight wear on trolley guides | Monitor |
| Business Class (16 seats) | Good | Seat 2A: armrest rattle | Rectify before service |
| Economy Class (132 seats) | Fair | Seat 14F: IFE screen blank; Seat 22B: pocket torn | Rectify IFE; replace pocket |
| Aft Galley (G2) | Good | None | None |
| Forward Lavatory | Fair | Waste flap hinge stiff | Rectify |
| Mid / Aft Lavatories (L2–L5) | Good | None | None |
| Overhead Bins | Good | Minor scuffing on door edges | Acceptable |
| Carpet – Forward | Good | No staining | None |
| Carpet – Economy | Fair | Minor wear rows 15–18 | Acceptable per Return Conditions |
| Sidewall Panels | Good | 2 panels with hairline cracks | Monitor |
| Emergency Equipment | Serviceable | All counts correct; no expiry within 12 months | None |

## 8. EXTERIOR CONDITION

| Zone | Condition | Noted Defects |
| --- | --- | --- |
| Nose / Radome | Good | Paint chip 1.5 cm at lower radome — no structural damage |
| Forward Fuselage (Sec 41/43) | Good | None |
| Mid Fuselage (Sec 44/46) | Good | Door R2 outer seal slight extrusion — serviceable |
| Aft Fuselage (Sec 46/48) | Good | None |
| Tail Section / APU Bay | Good | None |
| Vertical Stabiliser | Good | None |
| Horizontal Stabilisers | Good | None |
| Left Wing | Good | 2 small paint scuffs (approx 3×5 cm total) |
| Right Wing | Good | None |
| Engine No. 1 Nacelle | Good | None |
| Engine No. 2 Nacelle | Good | None |
| Overall Paint | Good | ~5% minor wear; no primer visible; no missing >2 cm |

## 9. DISCREPANCY LOG

| Ref | ATA | Description | Responsibility | Resolution |
| --- | --- | --- | --- | --- |
| DCR-001 | ATA 76 | Engine No.1 thrust lever friction high (8 lbf vs 6 lbf nominal) | Lessor to rectify pre-delivery | Adjust cable tension per AMM 76-11-00 |
| DCR-002 | ATA 25 | Seat 2A armrest rattle — Business Class | Lessee within 30 days | Replace armrest fitting; MEL Cat C |
| DCR-003 | ATA 25 | Seat 14F IFE screen blank | Lessee within 30 days | Replace IFE LRU |
| DCR-004 | ATA 25 | Seat 22B seatback pocket torn | Lessee within 60 days | Replace pocket fabric |
| DCR-005 | ATA 38 | Forward lavatory waste flap hinge stiff | Lessee within 30 days | Lubricate/replace hinge per AMM |
| DCR-006 | ATA 33 | Row 21 reading light inoperable — open MEL Cat C | Lessee: open MEL acknowledged | Replace at next A-Check |

## 10. OPEN MEL ITEMS AT DELIVERY

| MEL Ref | ATA | Description | Cat | Interval Expires |
| --- | --- | --- | --- | --- |
| MEL-23-001 | ATA 23 | SELCAL code channel 4 inoperable | B | 10 days from delivery |
| MEL-25-007 | ATA 25 | Seat 14F IFE screen blank | C | 30 days from delivery |
| MEL-33-012 | ATA 33 | Row 21 reading light inoperable | C | 120 days from delivery |

## 11. TECHNICAL RECORDS COMPLETENESS

| Record | Present | Complete | Comments |
| --- | --- | --- | --- |
| Aircraft Logbook | Yes | Yes | All entries legible and continuous |
| Engine No. 1 Logbook | Yes | Yes | Shop visit records attached |
| Engine No. 2 Logbook | Yes | Yes | Shop visit records attached |
| APU Logbook | Yes | Yes | All entries complete |
| Component Status List | Yes | Yes | Excel format; as at delivery date |
| LLP Tracking Sheets (Eng 1 & 2) | Yes | Yes | Disc sheets attached |
| AD Compliance Record | Yes | Yes | All ADs listed; repetitive due dates confirmed |
| SB Status Record | Yes | Yes | Alert SBs highlighted |
| C-Check Work Package | Yes | Yes | 1,847 work cards archived |
| Engine SV Reports | Yes | Yes | Delta TechOps (Eng 1), HAECO Xiamen (Eng 2) |
| Weight & Balance Report | Yes | Yes | Current; dated within 12 months |
| STCs / FAA 337s | Yes | Yes | IFE STC SA11801SW, SATCOM STC SA10912SW |

IN WITNESS WHEREOF the parties have executed this document as of the date first written above.

| LESSOR REPRESENTATIVE | LESSEE REPRESENTATIVE |
| --- | --- |
| By: _________________________ | By: _________________________ |
| Name: Thomas Kavanaugh | Name: Robert A. Fitch |
| Title: Technical Inspector | Title: Director of Technical |
| Date: 2022-11-01 | Date: 2022-11-01 |