HORIZON LEASING PARTNERS LP   |   AIRCRAFT LEASING

MAINTENANCE RESERVE CLAIM

Engine PRSV — ESN ESN-654321

Reference: MRC-N45678-2025-03

| Aircraft Type | MSN | Registration | Engines | Year |
| --- | --- | --- | --- | --- |
| Airbus A321-200 | 9102 | N45678 | CFM56-5B3/P | 2020 |

| Role | Party | Jurisdiction |
| --- | --- | --- |
| Claimant (Lessee) | Pacific Rim Air Travel Ltd. | USA |
| Reserve Account Holder | Horizon Leasing Partners LP | Ireland |
| MRO Facility | Lufthansa Technik AG | Hamburg, Germany |

CLAIM SUBMISSION

Dated: 2024-07-01

This document is strictly confidential and is intended solely for the named parties. Any reproduction, disclosure, or distribution without the prior written consent of Horizon Leasing Partners LP is strictly prohibited.

## 1. CLAIM SUMMARY

| Parameter | Detail |
| --- | --- |
| Claim Reference | MRC-N45678-2025-03 |
| Aircraft | Airbus A321-200 / N45678 / MSN 9102 |
| Engine | CFM56-5B3/P / ESN ESN-654321 (Position 1) |
| Event Type | Performance Restoration Shop Visit (PRSV) |
| MRO Facility | Lufthansa Technik AG, Hamburg — EASA 145.0001 / FAA OM3R164L |
| Induction Date | 2025-01-08 |
| Completion Date | 2025-03-22 |
| Aircraft back in service | 2025-03-28 |
| Claim Submission Date | 2025-04-10 |
| Total Claim Amount | USD 1,117,570 |
| MR Account Balance (Performance sub-account) | USD 824,000 |
| MR Account Balance (LLP sub-account) | USD 416,000 |
| Net Lessee Shortfall | USD 0 (accounts cover full claim) |

## 2. REASON FOR SHOP VISIT

The engine was inducted for a Performance Restoration shop visit following sustained EGT margin deterioration and a high-pressure compressor stall event on 2024-12-18. EGT margin had degraded to 12°C against a monitoring threshold of 20°C. An on-wing borescope conducted on 2024-12-22 revealed:

| Ref | ATA | Finding | Severity |
| --- | --- | --- | --- |
| BSI-001 | 72-30 | HPT Stage 1 blades: 4 blades with tip erosion >AMM limits (>0.15 in) | MANDATORY REPAIR |
| BSI-002 | 72-30 | HPT Stage 2 blades: 2 blades with leading edge nicks within limits — monitor | MONITOR |
| BSI-003 | 72-40 | LPT Stage 4 Nozzle: 3 segments with oxidation burns exceeding AMM limits | MANDATORY REPAIR |
| BSI-004 | 72-20 | HPC stages 5–7: performance coating loss contributing to EGT deterioration | PRICED WORKSCOPE |
| BSI-005 | 72-10 | Fan blades: all within AMM limits; no damage | SERVICEABLE |
| BSI-006 | 72-50 | No.1 and No.2 bearing compartment seals within limits | SERVICEABLE |

## 3. AGREED WORKSCOPE

| Task | ATA | Work Description | Qty | Status |
| --- | --- | --- | --- | --- |
| WS-01 | 72-00 | Complete engine disassembly; module split into 5 modules | N/A | Done |
| WS-02 | 72-10 | Fan module: clean, inspect; restore tip clearances; replace 4 damaged fan blades | 4 blades | Done |
| WS-03 | 72-20 | LPC booster: disassembly; clean; inspect; replace full blade set Stage 2 | Full set Stg 2 | Done |
| WS-04 | 72-20 | HPC: disassembly; SWET process on discs; replace HVOF coating Stages 5–7 | Coating renewed | Done |
| WS-05 | 72-20 | HPC Disc Stage 3: replace (CSN at 14,990 — within 10 of limit per LLP sheet) | 1 disc | Done |
| WS-06 | 72-30 | Combustion case: clean; inspect; 2 segments repaired per SRM 72-41-00 | 2 repairs | Done |
| WS-07 | 72-30 | HPT Stage 1 blades: full replacement set — all new, RFN traceable | Full set (80) | Done |
| WS-08 | 72-30 | HPT Stage 2 blades: replace 2 blades exceeding AMM limits | 2 blades | Done |
| WS-09 | 72-30 | HPT Stage 1 Nozzle Guide Vanes: full ring replacement | Full ring | Done |
| WS-10 | 72-30 | HPT Stage 2 Nozzle Guide Vanes: full ring replacement | Full ring | Done |
| WS-11 | 72-40 | LPT Stage 4 Nozzle: 6 segments repaired per SRM; remaining 3 replaced | 3 segments | Done |
| WS-12 | 72-40 | LPT blades Stages 4–7: clean, inspect; replace 8 blades (FOD/erosion) | 8 blades | Done |
| WS-13 | 72-50 | No. 1 Bearing: replace oil seals and carbon seals; bearing inspected (no replacement) | Seals replaced | Done |
| WS-14 | 72-50 | No. 3 Bearing: replace (at CSN limit per LLP sheet) | 1 bearing | Done |
| WS-15 | 73-00 | Fuel nozzles: replace all 20 (atomisation degraded) | 20 nozzles | Done |
| WS-16 | 74-00 | Igniter plugs: replace both (hours on limit) | 2 plugs | Done |
| WS-17 | 72-00 | Module reassembly; cold and hot section balance | N/A | Done |
| WS-18 | 72-00 | Performance test cell — 3 runs; EGT margin confirmed at 62°C — PASS | N/A | PASS |
| WS-19 | 72-00 | EASA Form 1 / FAA 8130-3 release; QA acceptance | N/A | Done |

## 4. PARTS LIST

| Part No. | Description | Qty | Cond. | Cert. Ref. | Cost |
| --- | --- | --- | --- | --- | --- |
| 1794M91P01 | Fan Blade set (4 replaced) | 4 | NEW OEM | CFM/BOR2024 | USD 42,800 |
| 1795M11P01 | LPC Blade set Stage 2 (28 blades) | 28 | NEW OEM | LHT/BOR2025 | USD 28,600 |
| 1796M01P01 | HPC Disc Stage 3 (LLP at limit) | 1 | NEW OEM | CFM/LLP2025 | USD 148,500 |
| 1797M03P01 | HPT Stage 1 Blade set (80 blades) | 80 | NEW OEM | LHT/BOR2025 | USD 312,400 |
| 1797M04P01 | HPT Stage 2 Blade (2 blades) | 2 | NEW OEM | LHT/BOR2025 | USD 18,600 |
| HPT-NGV-S1 | HPT Stg 1 Nozzle Guide Vane Ring | 1 | NEW OEM | CFM/NGV2025 | USD 89,200 |
| HPT-NGV-S2 | HPT Stg 2 Nozzle Guide Vane Ring | 1 | NEW OEM | CFM/NGV2025 | USD 72,100 |
| LPT-NOZ-S4 | LPT Stage 4 Nozzle (3 segments) | 3 | NEW OEM | LHT/BOR2025 | USD 34,500 |
| LPT-BLD-MIX | LPT Blades Stages 4–7 (8 blades) | 8 | NEW OEM | LHT/BOR2025 | USD 22,800 |
| BRG-NO3-XX | No. 3 Bearing (LLP at limit) | 1 | NEW OEM | CFM/BRG2025 | USD 8,400 |
| SEL-CRBNS-1 | Carbon Seals No.1 Bearing Compartment | 3 | NEW OEM | LHT/SL2025 | USD 4,200 |
| FNZ-20PLCE | Fuel Nozzles (set of 20) | 20 | NEW OEM | CFM/FN2025 | USD 41,600 |
| IGN-PLUG-SET | Igniter Plug Set (2×) | 2 | NEW OEM | LHT/IG2025 | USD 1,800 |
| HVOF-COAT-567 | HVOF Coating HPC Stages 5–7 | 3 stages | PROCESS | LHT/HVOF2025 | USD 36,800 |
| MISC-HARDWARE | Hardware (nuts, bolts, seals, gaskets) | Lot | NEW OEM | LHT/MISC2025 | USD 12,400 |

## 5. LABOUR BREAKDOWN

| Activity | Hours | Rate (USD/hr) | Total (USD) |
| --- | --- | --- | --- |
| Engine Disassembly & Assembly — Lead Mechanic | 284 | USD 145 | USD 41,180 |
| Module Build Fan/LPC/HPC — Mechanic | 312 | USD 132 | USD 41,184 |
| Hot Section Build HPT/LPT — Mechanic | 298 | USD 132 | USD 39,336 |
| NDT (FPI/UT/Eddy Current) | 96 | USD 158 | USD 15,168 |
| QA Inspection | 48 | USD 165 | USD 7,920 |
| Test Cell Operator (3 runs) | 36 | USD 145 | USD 5,220 |
| Documentation / Certification | 24 | USD 120 | USD 2,880 |
| Overhead (15%) | — | — | USD 23,082 |
| TOTAL LABOUR | 1,098 hrs | — | USD 175,970 |

## 6. COST SUMMARY AND ELIGIBILITY

| Category | Amount | Eligible Account | Eligible? |
| --- | --- | --- | --- |
| OEM Parts — HPT/NGV/Fan/Fuel | USD 876,500 | Engine Performance | YES |
| HPC Stage 3 LLP replacement | USD 148,500 | Engine LLP | YES |
| No. 3 Bearing LLP replacement | USD 8,400 | Engine LLP | YES |
| Labour (1,098 hrs) | USD 175,970 | Engine Performance | YES |
| Test Cell (3 runs) | USD 38,500 | Engine Performance | YES |
| Transport (ferry + delivery) | USD 14,200 | Engine Performance | YES |
| Tooling | USD 12,400 | Engine Performance | YES |
| TOTAL CLAIM | USD 1,117,570 | — | — |
| Performance sub-account balance | USD 824,000 | — | — |
| LLP sub-account balance | USD 416,000 | — | — |
| Lessee shortfall | USD 0 | — | COVERED |

## 7. SUPPORTING DOCUMENTATION ATTACHED

| Ref | Document | Pages |
| --- | --- | --- |
| SUPP-01 | LHT Shop Visit Work Order WO-LHT-2025-04812 | 48 |
| SUPP-02 | Teardown Inspection Report (TIR) | 32 |
| SUPP-03 | Build-Up Inspection Report (BIR) | 28 |
| SUPP-04 | Performance Test Cell Run Report — Runs 1, 2, 3 | 18 |
| SUPP-05 | EASA Form 1 / FAA 8130-3 for all replaced parts | 31 |
| SUPP-06 | Parts traceability back-to-birth documentation | 22 |
| SUPP-07 | Updated LLP tracking sheet (post-PRSV) | 4 |
| SUPP-08 | Updated Engine Logbook pages | 8 |
| SUPP-09 | Final Invoice — Lufthansa Technik AG | 6 |
| SUPP-10 | Photo documentation (188 photos) | 38 |
| SUPP-11 | Pre-PRSV borescope report (Delta TechOps) | 14 |

IN WITNESS WHEREOF the parties have executed this document as of the date first written above.

| CLAIMANT (LESSEE) | LESSOR (APPROVAL) |
| --- | --- |
| By: _________________________ | By: _________________________ |
| Name: Sarah J. Watkins | Name: Patrick Dunne |
| Title: VP Technical Operations | Title: Portfolio Technical Manager |
| Date: 2025-04-10 | Date: ___________ |