ATLAS AIRCRAFT TRUST   |   AIRCRAFT LEASING

MAINTENANCE RESERVE CLAIM

Engine PRSV — ESN ESN-100201

Reference: MRC-N56789-2025-04

| Aircraft Type | MSN | Registration | Engines | Year |
| --- | --- | --- | --- | --- |
| Boeing 737 MAX 8 | 45912 | N56789 | CFM LEAP-1B27 | 2022 |

| Role | Party | Jurisdiction |
| --- | --- | --- |
| Claimant (Lessee) | Nordic Skies AS | USA |
| Reserve Account Holder | Atlas Aircraft Trust | Ireland |
| MRO Facility | Lufthansa Technik AG | Hamburg, Germany |

CLAIM SUBMISSION

Dated: 2024-09-01

This document is strictly confidential and is intended solely for the named parties. Any reproduction, disclosure, or distribution without the prior written consent of Atlas Aircraft Trust is strictly prohibited.

## 1. CLAIM SUMMARY

| Parameter | Detail |
| --- | --- |
| Claim Reference | MRC-N56789-2025-04 |
| Aircraft | Boeing 737 MAX 8 / N56789 / MSN 45912 |
| Engine | CFM LEAP-1B27 / ESN ESN-100201 (Position 1) |
| Event Type | Performance Restoration Shop Visit (PRSV) |
| MRO Facility | Lufthansa Technik AG, Hamburg — EASA 145.0001 / FAA OM3R164L |
| Induction Date | 2025-01-08 |
| Completion Date | 2025-03-22 |
| Aircraft back in service | 2025-03-28 |
| Claim Submission Date | 2025-04-10 |
| Total Claim Amount | USD 1,117,570 |
| MR Account Balance (Performance sub-account) | USD 824,000 |
| MR Account Balance (LLP sub-account) | USD 416,000 |
| Net Lessee Shortfall | USD 0 (accounts cover full claim) |

## 2. REASON FOR SHOP VISIT

The engine was inducted for a Performance Restoration shop visit following sustained EGT margin deterioration and a high-pressure compressor stall event on 2024-12-18. EGT margin had degraded to 12°C against a monitoring threshold of 20°C. An on-wing borescope conducted on 2024-12-22 revealed:

| Ref | ATA | Finding | Severity |
| --- | --- | --- | --- |
| BSI-001 | 72-30 | HPT Stage 1 blades: 4 blades with tip erosion >AMM limits (>0.15 in) | MANDATORY REPAIR |
| BSI-002 | 72-30 | HPT Stage 2 blades: 2 blades with leading edge nicks within limits — monitor | MONITOR |
| BSI-003 | 72-40 | LPT Stage 4 Nozzle: 3 segments with oxidation burns exceeding AMM limits | MANDATORY REPAIR |
| BSI-004 | 72-20 | HPC stages 5–7: performance coating loss contributing to EGT deterioration | PRICED WORKSCOPE |
| BSI-005 | 72-10 | Fan blades: all within AMM limits; no damage | SERVICEABLE |
| BSI-006 | 72-50 | No.1 and No.2 bearing compartment seals within limits | SERVICEABLE |

## 3. AGREED WORKSCOPE

| Task | ATA | Work Description | Qty | Status |
| --- | --- | --- | --- | --- |
| WS-01 | 72-00 | Complete engine disassembly; module split into 5 modules | N/A | Done |
| WS-02 | 72-10 | Fan module: clean, inspect; restore tip clearances; replace 4 damaged fan blades | 4 blades | Done |
| WS-03 | 72-20 | LPC booster: disassembly; clean; inspect; replace full blade set Stage 2 | Full set Stg 2 | Done |
| WS-04 | 72-20 | HPC: disassembly; SWET process on discs; replace HVOF coating Stages 5–7 | Coating renewed | Done |
| WS-05 | 72-20 | HPC Disc Stage 3: replace (CSN at 14,990 — within 10 of limit per LLP sheet) | 1 disc | Done |
| WS-06 | 72-30 | Combustion case: clean; inspect; 2 segments repaired per SRM 72-41-00 | 2 repairs | Done |
| WS-07 | 72-30 | HPT Stage 1 blades: full replacement set — all new, RFN traceable | Full set (80) | Done |
| WS-08 | 72-30 | HPT Stage 2 blades: replace 2 blades exceeding AMM limits | 2 blades | Done |
| WS-09 | 72-30 | HPT Stage 1 Nozzle Guide Vanes: full ring replacement | Full ring | Done |
| WS-10 | 72-30 | HPT Stage 2 Nozzle Guide Vanes: full ring replacement | Full ring | Done |
| WS-11 | 72-40 | LPT Stage 4 Nozzle: 6 segments repaired per SRM; remaining 3 replaced | 3 segments | Done |
| WS-12 | 72-40 | LPT blades Stages 4–7: clean, inspect; replace 8 blades (FOD/erosion) | 8 blades | Done |
| WS-13 | 72-50 | No. 1 Bearing: replace oil seals and carbon seals; bearing inspected (no replacement) | Seals replaced | Done |
| WS-14 | 72-50 | No. 3 Bearing: replace (at CSN limit per LLP sheet) | 1 bearing | Done |
| WS-15 | 73-00 | Fuel nozzles: replace all 20 (atomisation degraded) | 20 nozzles | Done |
| WS-16 | 74-00 | Igniter plugs: replace both (hours on limit) | 2 plugs | Done |
| WS-17 | 72-00 | Module reassembly; cold and hot section balance | N/A | Done |
| WS-18 | 72-00 | Performance test cell — 3 runs; EGT margin confirmed at 62°C — PASS | N/A | PASS |
| WS-19 | 72-00 | EASA Form 1 / FAA 8130-3 release; QA acceptance | N/A | Done |

## 4. PARTS LIST

| Part No. | Description | Qty | Cond. | Cert. Ref. | Cost |
| --- | --- | --- | --- | --- | --- |
| 1794M91P01 | Fan Blade set (4 replaced) | 4 | NEW OEM | CFM/BOR2024 | USD 42,800 |
| 1795M11P01 | LPC Blade set Stage 2 (28 blades) | 28 | NEW OEM | LHT/BOR2025 | USD 28,600 |
| 1796M01P01 | HPC Disc Stage 3 (LLP at limit) | 1 | NEW OEM | CFM/LLP2025 | USD 148,500 |
| 1797M03P01 | HPT Stage 1 Blade set (80 blades) | 80 | NEW OEM | LHT/BOR2025 | USD 312,400 |
| 1797M04P01 | HPT Stage 2 Blade (2 blades) | 2 | NEW OEM | LHT/BOR2025 | USD 18,600 |
| HPT-NGV-S1 | HPT Stg 1 Nozzle Guide Vane Ring | 1 | NEW OEM | CFM/NGV2025 | USD 89,200 |
| HPT-NGV-S2 | HPT Stg 2 Nozzle Guide Vane Ring | 1 | NEW OEM | CFM/NGV2025 | USD 72,100 |
| LPT-NOZ-S4 | LPT Stage 4 Nozzle (3 segments) | 3 | NEW OEM | LHT/BOR2025 | USD 34,500 |
| LPT-BLD-MIX | LPT Blades Stages 4–7 (8 blades) | 8 | NEW OEM | LHT/BOR2025 | USD 22,800 |
| BRG-NO3-XX | No. 3 Bearing (LLP at limit) | 1 | NEW OEM | CFM/BRG2025 | USD 8,400 |
| SEL-CRBNS-1 | Carbon Seals No.1 Bearing Compartment | 3 | NEW OEM | LHT/SL2025 | USD 4,200 |
| FNZ-20PLCE | Fuel Nozzles (set of 20) | 20 | NEW OEM | CFM/FN2025 | USD 41,600 |
| IGN-PLUG-SET | Igniter Plug Set (2×) | 2 | NEW OEM | LHT/IG2025 | USD 1,800 |
| HVOF-COAT-567 | HVOF Coating HPC Stages 5–7 | 3 stages | PROCESS | LHT/HVOF2025 | USD 36,800 |
| MISC-HARDWARE | Hardware (nuts, bolts, seals, gaskets) | Lot | NEW OEM | LHT/MISC2025 | USD 12,400 |

## 5. LABOUR BREAKDOWN

| Activity | Hours | Rate (USD/hr) | Total (USD) |
| --- | --- | --- | --- |
| Engine Disassembly & Assembly — Lead Mechanic | 284 | USD 145 | USD 41,180 |
| Module Build Fan/LPC/HPC — Mechanic | 312 | USD 132 | USD 41,184 |
| Hot Section Build HPT/LPT — Mechanic | 298 | USD 132 | USD 39,336 |
| NDT (FPI/UT/Eddy Current) | 96 | USD 158 | USD 15,168 |
| QA Inspection | 48 | USD 165 | USD 7,920 |
| Test Cell Operator (3 runs) | 36 | USD 145 | USD 5,220 |
| Documentation / Certification | 24 | USD 120 | USD 2,880 |
| Overhead (15%) | — | — | USD 23,082 |
| TOTAL LABOUR | 1,098 hrs | — | USD 175,970 |

## 6. COST SUMMARY AND ELIGIBILITY

| Category | Amount | Eligible Account | Eligible? |
| --- | --- | --- | --- |
| OEM Parts — HPT/NGV/Fan/Fuel | USD 876,500 | Engine Performance | YES |
| HPC Stage 3 LLP replacement | USD 148,500 | Engine LLP | YES |
| No. 3 Bearing LLP replacement | USD 8,400 | Engine LLP | YES |
| Labour (1,098 hrs) | USD 175,970 | Engine Performance | YES |
| Test Cell (3 runs) | USD 38,500 | Engine Performance | YES |
| Transport (ferry + delivery) | USD 14,200 | Engine Performance | YES |
| Tooling | USD 12,400 | Engine Performance | YES |
| TOTAL CLAIM | USD 1,117,570 | — | — |
| Performance sub-account balance | USD 824,000 | — | — |
| LLP sub-account balance | USD 416,000 | — | — |
| Lessee shortfall | USD 0 | — | COVERED |

## 7. SUPPORTING DOCUMENTATION ATTACHED

| Ref | Document | Pages |
| --- | --- | --- |
| SUPP-01 | LHT Shop Visit Work Order WO-LHT-2025-04812 | 48 |
| SUPP-02 | Teardown Inspection Report (TIR) | 32 |
| SUPP-03 | Build-Up Inspection Report (BIR) | 28 |
| SUPP-04 | Performance Test Cell Run Report — Runs 1, 2, 3 | 18 |
| SUPP-05 | EASA Form 1 / FAA 8130-3 for all replaced parts | 31 |
| SUPP-06 | Parts traceability back-to-birth documentation | 22 |
| SUPP-07 | Updated LLP tracking sheet (post-PRSV) | 4 |
| SUPP-08 | Updated Engine Logbook pages | 8 |
| SUPP-09 | Final Invoice — Lufthansa Technik AG | 6 |
| SUPP-10 | Photo documentation (188 photos) | 38 |
| SUPP-11 | Pre-PRSV borescope report (Delta TechOps) | 14 |

IN WITNESS WHEREOF the parties have executed this document as of the date first written above.

| CLAIMANT (LESSEE) | LESSOR (APPROVAL) |
| --- | --- |
| By: _________________________ | By: _________________________ |
| Name: Sarah J. Watkins | Name: Patrick Dunne |
| Title: VP Technical Operations | Title: Portfolio Technical Manager |
| Date: 2025-04-10 | Date: ___________ |