GLOBAL WING FINANCE BV   |   AIRCRAFT LEASING

TECHNICAL ACCEPTANCE REPORT

Pre-Delivery Technical Inspection — N23456

Reference: TAR-N23456-2023-06-01

| Aircraft Type | MSN | Registration | Engines | Year |
| --- | --- | --- | --- | --- |
| Airbus A320-200 | 8841 | N23456 | CFM56-5B4/P | 2017 |

| Role | Party | Jurisdiction |
| --- | --- | --- |
| Lessor Inspector | Walsh Aviation Consulting Ltd | Ireland |
| Lessee Inspector | Meridian Express Corp. | USA |
| Independent | Aviation Analysts Group LLC | USA |

ACCEPTED — DELIVERY APPROVED

Dated: 2023-06-01

This document is strictly confidential and is intended solely for the named parties. Any reproduction, disclosure, or distribution without the prior written consent of Global Wing Finance BV is strictly prohibited.

## 1. INSPECTION TEAM

| Role | Name | Company | Qualifications | Scope |
| --- | --- | --- | --- | --- |
| Lessor Technical Rep | Patrick Walsh | Walsh Aviation Consulting | AME B1+B2, 25 yrs Boeing/CFM | Overall acceptance, records sign-off |
| Lessee Technical Rep | Dr. Marina Torres | VP Technical | PhD Aerospace, ex-Boeing | Lessee acceptance, squawk sign-off |
| Independent Inspector | George Nakamura | Aviation Analysts Group | A&P/IA, ex-FAA DER | Neutral verification |
| Engine Inspector | Sean Callaghan | CFM Authorized Service Center | CFM56 factory-trained, Part 145 | Engine borescope, LLP verification |
| Structural Inspector | Hui Zhang | AAG Structures | NDT Level III (UT/FPI/Eddy) | Structural, corrosion assessment |
| Records Specialist | Aoife Brennan | Walsh Aviation Consulting | 10 yrs records auditing | Technical records completeness |

## 2. GROUND INSPECTION RESULTS

| # | Area | Finding | Status | Action |
| --- | --- | --- | --- | --- |
| GI-01 | Nose / Radome | Radome crack-free; paint chip 1.2 cm lower skirt — no structural significance | ACCEPTABLE | Touch-up |
| GI-02 | Flight Deck | All panels complete; co-pilot EFIS brightness slightly low | ACCEPTABLE | Adjust EFIS brightness |
| GI-03 | Forward Lavatory | All systems functional | PASS | None |
| GI-04 | Business Class | 168 seats inspected; 3 defects noted | MINOR DEFECTS | See squawk list |
| GI-05 | Aft Galley | All ovens/coffeemakers functional | PASS | None |
| GI-06 | Emergency Equipment | All in-date; counts correct | PASS | None |
| GI-07 | Cargo Compartments | Lining intact; no corrosion; door seals good | PASS | None |
| GI-08 | Nose Landing Gear | No corrosion; microswitches OK; tyre 70% worn — replace pre-delivery | ACTION | Replace NLG tyres before delivery |
| GI-09 | Left MLG | No corrosion; brake wear 45%; tyre 55% | ACCEPTABLE | Replace per Lessee programme |
| GI-10 | Right MLG | No corrosion; brake wear 38%; tyre 60% | ACCEPTABLE | None |
| GI-11 | Left Wing Lower | Sealant aging J10-L; slight seepage cold soak test | ACTION | Re-seal J10-L before delivery |
| GI-12 | Left Wing Upper | Spoilers 6×: functional; ailerons: functional; no disbonds | PASS | None |
| GI-13 | Right Wing | As Left Wing — all satisfactory | PASS | None |
| GI-14 | Engine No.1 Nacelle | Fan cowl OK; inlet no FOD; reverser seals good | PASS | None |
| GI-15 | Engine No.2 Nacelle | As Engine No.1 | PASS | None |
| GI-16 | APU Bay | No oil leaks; exhaust duct clear | PASS | None |
| GI-17 | Fuselage Upper | No cracks; 3 minor repairs within SRM limits | PASS | None |
| GI-18 | Fuselage Lower | 2 SRM repairs complete; paperwork complete | PASS | None |
| GI-19 | Empennage | No delaminations; trim tab functional; all static wicks present | PASS | None |
| GI-20 | Exterior Paint | Condition ~85%; no primer visible; marks correct | PASS | None |

## 3. BORESCOPE — ENGINE NO. 1 (ESN ESN-771201)

| Module | Finding | Limit | Status |
| --- | --- | --- | --- |
| Fan (24 blades) | All within AMM; 2 blades: leading edge micro-nicks within 0.005 in limit | 0.010 in | PASS |
| LPC Booster | All serviceable | Per AMM 72-21-00 | PASS |
| HPC Stg 3 | CSN at 14,990/15,000 — mandatory replacement at next SV per LLP sheet | 15,000 FC | NOTED — approaching limit |
| HPC Stg 4–9 | Serviceable; coating wear within AMM; Stg 5 at 62% life | Per AMM | PASS |
| Combustion Liner | Slight hot spot burn 2.5 cm — within SRM limits | SRM 72-41-00 | PASS |
| HPT Stg 1 blades | 3 blades tip erosion at 80% of AMM limit — monitor | AMM 72-31-00 | MONITOR |
| HPT Stg 2 blades | All within AMM limits | — | PASS |
| LPT Stg 4 Nozzle | All within limits; oxidation present but allowable | SRM 72-53-00 | PASS |
| LPT Stg 4–7 blades | All serviceable | Per AMM | PASS |
| No. 1 Bearing | Carbon seals serviceable; no carbon dust migration | Per AMM 72-51-00 | PASS |

## 4. BORESCOPE — ENGINE NO. 2 (ESN ESN-771202)

| Module | Finding | Status |
| --- | --- | --- |
| Fan | All blades within limits; no FOD | PASS |
| LPC | All blades serviceable | PASS |
| HPC Stg 3 | CSN 14,786 — within limit, approaching | PASS / MONITOR |
| Combustion | Clean burn pattern; within limits | PASS |
| HPT | Stg 1 blade erosion <50% AMM limit | PASS |
| LPT | All stages serviceable | PASS |
| Bearings | No abnormalities | PASS |

## 5. ACCEPTANCE FLIGHT

| Parameter | Detail |
| --- | --- |
| Flight Date | 2023-06-01 |
| Route | DFW–DAL–DFW (acceptance circuit, 55 min airborne) |
| Commander | Capt. Tom O'Brien (ATP, B737) — Lessor; Capt. Alicia Fernandez (ATP) — Lessee |
| Weather | VFR; wind 180/12 kts; vis 10+; temp 22°C |
| Engine No.1 EGT margin (T/O derate 5%) | 36°C — PASS (min 30°C) |
| Engine No.2 EGT margin | 40°C — PASS |
| APU in flight | Shut down post engine start; restarted in cruise for bleed test — PASS |
| Pressurisation | Normal; cabin alt 6,200 ft at FL360 |
| Fuel burn | Within 2% of WBM prediction |
| Autoland | ILS RWY 18L DFW; Cat I autoland; rollout normal |
| All systems | Hydraulics, electrical, avionics, ACARS, FMC, autopilot — all PASS |
| Acceptance Decision | ACCEPTED — Delivery Acceptance Certificate signed at gate |

## 6. PRE-DELIVERY ACTION ITEMS

| Item | Description | Responsibility | Due |
| --- | --- | --- | --- |
| PD-01 | Replace NLG tyres (GI-08) | Lessor | By Delivery Date |
| PD-02 | Re-seal wing panel J10-L (GI-11) | Lessor | By Delivery Date |
| PD-03 | Cosmetic paint touch-up radome skirt (GI-01) | Lessor — optional | Delivery + 30 days |

All cabin squawks (SQK-01 through SQK-05) were rectified prior to the acceptance flight and are confirmed closed.

IN WITNESS WHEREOF the parties have executed this document as of the date first written above.

| LESSOR INSPECTOR | LESSEE INSPECTOR | INDEPENDENT |
| --- | --- | --- |
| By: _________________________ | By: _________________________ | By: _________________________ |
| Name: Patrick Walsh | Name: Dr. Marina Torres | Name: George Nakamura |
| Title: Senior Technical Inspector | Title: VP Technical | Title: Aviation Analysts Group |
| Date: 2023-06-01 | Date: 2023-06-01 | Date: 2023-06-01 |